Program guts:

Startup:

+24V off

+5V/+12V off

Computer on.

Start program.

Program fetches start up parameters from file. This should not be obvious to the operator but hidden where only the wise can find.

- Polulu values, assignments  Does +5 V have to be on to get these registered?

  - 2.3 ; polulu center value X side

  - 2. ;polulu P value X side

  - 1.5; polulu I value X side

  - 3.0; polulu D value X side

  - 2.4; polulu park voltage x side

  - 2.3 ; polulu center value Y side

  - 2. ;polulu P value Y side

  - 1.5; polulu I value Y side

  - 3.0; polulu D value Y side

  - 2.4; polulu park voltage Y side

  - 0.0; off voltage (both)

- Stepper motor values

  - Steps per rotation X Can, (- sign to reverse sense of motor)

  - Steps per rotation Y Can, (- sign to reverse sense of motor)

  - Max acceration cans (absolute)

  - Max velocity  cans (absolute)

  - Steps per uM X, (- sign reverses sense of motor)

  - Steps per uM Y, (- sign reverses sense of motor)

  - Max acceleration stage  (absolute)

  - Max velocity stage  (absolute)

  - Standby power X can, Y can

  - Standby power X stage, Y stage

  - Standby power relaxation time stages

  - Standby power relaxation time cans.

  - Micro steps X can, Y can

  - Micro steps X stage, Y  stage

- Input parameters

  - Foot pedal pot to can speed (fast)  constant

  - Foot pedal to can speed (slow)

  - Joystick to can speed  (fast)

  - Joystick to can speed (slow)

  - Joystick to can X motion (fast)

  - Joystic to X motion (slow)

- System parameters

  - Bump point X  (point where fiber is moved) should be 0 to start for both

  - Bump point Y

  - Movement of bump point per layer  X

  - Movement of bump point per layer Y

  - Odd minus even count ( should be 1 but could be 0)  for std wind 1.

  - Progressive count ( 0 but could be 1) used to wind V coils.

- Enter the button assignments. These will likely change.  Many won’t be assigned.  A second box will probably be built with fewer buttons.

INSERT     TABLE       HERE

- Zero parameters and load the above parameters.

- Set Polulu’s to 0 volts.

- Turn on +5/+12V the +24V.

- Load stepper parameters.  Logic comes from +24 so we have to have +24 to make it go.

Get coil parameters:

- Provide list of file names of existing coils.  ( This should start with NEW as the only  entry).     User selects the one of choice.  Cursor selection?

- Load, save, and all the coil parameters are presented as options.

- Coil parameters.

  - Hub diameter.  Presented in inches and cm.  If one is changed, the other follows. E.G. 2 in.

  - Coil height.   Presented in inches and cm.  If one is changed, the other follows.  E.G 0.7 in

  - Fiber diameter in um ,  e.g.  165.  Range (80-400 um)

  - Number of turns, base layer =n.  User enters a number.  Beside it is “calculated base layer turns  =”  b value/ c value.   E.G 200

  - Number of layers. (N)   e.g. 32

  - Calculated coil OD.  = Hub diameter +fiber diameter *( 1 + SQRT(3)/2*(N-1)) We don’t enter this but it is calculated for the user’s benefit. And shown for reality check.  SHOW IN INCHES AND CM.

  - Coil type: 1= standard quadrupole, 2= filler climber quadrupole, 3= all layers get the same number of turns. 4= flat wind ( single ended), 5=V wind, 6= multi segment quadrupole.    Only 1 and 2 are likely to implemented in the near future.

- If save, the list of current names appears.  There is a choice of one of the current names or a new name.  Saving under a current name requires the “are you sure?” question and the replaced name is pushed into an archive name.

- If load is pushed, the parameters are loaded into the program. And we enter the set up phase.

SET UP: FIBER LOADING. & Base layer.

- The servo motors are off.

- Both brakes off.   Button push.

- Position X & Y stages.  X is on the left.

  - Joystick using the active stage

  - May use the fast and slow toggling button.

  - Button push to get the inactive stage to respond to the joystick.

  - Joystick that stage into position.

- Install and position tooling between shafts.

- Right brake on.    button

- Bring the cans to TDC.

  - Joystick the  active can.

  - May use fast and slow toggling button.

  - May record TDC by button push for this side here or later.

  - Toggle joystick into controlling the inactive can.

  - Bring inactive can to TDC by joystick.

  - Record these positions as TDC. Button push

- The left side payout is aligned with the left side of the spool.

  - Toggle to joystick left side active

  - Joystick control into position.

- The right side payout is aligned with the left side of the spool.

  - Toggle to joystick inactive side control.

  - May use fast and slow motion of joystick.

  - This is Y start 0.  Position will be more certain when there is fiber in the machine.

- The spools are threaded into the cans.

- One payout spool is pinned; the one on the left.

- Fiber is threaded onto left tensioner, to idler to payout  to spool over to the right payout  idler, tensioner and then to the  other payout spool.  The right tensioner  servo is turned on.  Button. Push.

- Reset the rev counter , right side , button.

- Set lead/lag of guide.  ( right stage)  Lead/lag must be displayed on monitor in units of fiber diameters as lag from the edge is forced to 0.  Lag is negative and lead is positive.  Lead and lag are defined as the position of the fiber WRT to direction of motion of the payout guide wheel.  If the payout wheel is in advance of the point of fiber application you are in a lead condition.

- Move  to park.

  - Rt. Brake on.

  - Lf. Brake off.

  - By joystick move active stage.  Record this position as X park. (Exception to the active / inactive nomenclature) Button.

  - Lf brake on

  - Rt. Brake on

- Start winding base layer.

  - Press start wind button.  If lead,  inactive stage moves to start position plus lead lead.  If lag, inactive stage moves to start . [computer control] At end of each  turn, the Y stage position is incremented by 1 fiber diameter. [computer]  If the position is positive the active stage goes there if negative, no motion is made. (Eventually the lag will be taken out and the  stage will move.  Initially lead is to the right and lag is to the left on even numbered layers this is reversed.)

  - foot pedal makes the active (left) can rotate. and the stage is slaved to move 1 fiber diameter per revolution.  Wind to the end.  If too few turns  wind more turns.  If too many and they don’t all fit, don’t do all the turns.   This will be because the fiber diameter is nominal and perhaps not quite right. Some smoothing is required.  Joystick movement of the cans is required.

  - Computer counts the turns as they are made.

- Active stage motion stops when it has moved the height of the coil from the start. (inclusive of the lead lag)

- Number of turns is reset to the number actually put on the hub.  Button push.

- One of these will happen:

  - The base layer will be taken off.  The fiber will be put back on the payout spool. The guide will return to the start.  The mandrel is painted with  epoxy.  The first layer is then wound for real.

    - In unwind mode the can is rotated in the other directed

    - The direction of the guide is reversed.

    - The servo is unchanged..

  - The fiber will be dismounted and we stop the winding.  This is the worst case scenario…save broken fiber.

  - We will paint epoxy onto the layer just wound and continue.

- If in climber/filler mode, wind first turn of next layer.  If in “ true quadrupole” mode don’t.  Keep record of the turns.

- Record keeping consists of.  Counting the total turns on the layer, counting the layer.

SECOND LAYER, THIRD LAYER  and all additional layers.

- Move the inactive stage and can to Park.  (  Record this position. On second layer only)

  - Active brake on

  - Inactive brake off

  - Button toggle joystick to inactive stage

  - Move stage to park joystick

  - Put inactive servo in park.

- Set active servo to park tension. Button push.

- Inactive brake on, active brake off.

- Swap ends, (rotate whole machine)   Now X is on the right;  active and inactive are reversed.  Increase layer count by 1

- Set  new inactive tensioner  to servo.  (New will no longer be used)

- Remove spool lock pin.

- Move the inactive stage to the edge of the spool. (Record this number. Second  layer only)

- Start winding.  If lag or lead follow rules in #16 before.

- Wind  K turns. K= N-1 (because it is even layer) and  -1 if in climber/filler mode as the end fiber is already there.

- Increment layer counter.  This resets the turn counter.

- Start winding to the right.  Wind  M turns. M= N + (-1 on left for filler and  + 1 on the other side for a filler)  Otherwise just wind M.  In one case layer 3 gets one less turn from this winding end and the next layer gets a turn.

- Move inactive stage to park.  This position was recorded in base layer operation.

LAST LAYER.

- Wind last layer like layer 2.

- Active fiber end is taped to the coil winding tooling flange.

- Active end servo is turned off.  Sufficient lead fiber is removed from pay out spool and cut.

- Other end is taped where the fiber crosses the flange.

- Servo is turned off and sufficient fiber is removed from pay out spool.

- Both brakes off.

- Remove coil.

- Stages may have to be moved apart.

Save the build file:

Additional entries to file are:

- Winder’s name

- Coil number

- Start time

- Stop time

- New base layer count

- Layers

- Type of wind

- Epoxy lot and exp date.

- Fiber   Type and Lot

- Coil type (generic name)

MACROS:  It is forseen that certain sets of buttons will be used always in conjunction and they will be made into macro buttons.  Later for this.

How things work by naming.

X and Y are sides of the machine.  X and Y can be stamped onto the parts of the machine.

Active and Inactive change with the rotation of the machine.   Active Stage and Rotator are on the left.  Note that the point of fiber application.  The coil moves in Z and theta and the point of application is fixed in space.  The lens is always looking at the point of application.

All buttons are referenced to active and inactive NOT X and Y. However, most positions are in the X and Y system.

This is a first draft and it is certain to have errors.  If something appears inconsistent, it probably s wrong.

We, Steve and I will work this out Friday.  More likely we will get it better but not exactly right.